Universidad de Puerto Rico

Recinto Universitario de Mayagüez

# INEL 4206 – Microprocesadores

## Primavera 2002

Ejercicios de práctica

Examen Parcial I

- Processor Implementation. Add an instruction BrL (Branch and link)  to the Easy I processor designed in class.  You must provide any necessary additions/modifications to the Easy I datapaths, flowcharts and state transition table.  The intruction has the following functionality:

| Symbolic Name | Assembler Example | Action |
| --- | --- | --- |
| BrL | BrL sort | MEM[X-2] <- PC+2 PC <- X |

- Processor Implementation. Add an instruction RtL (Return using link) to the Easy I processor designed in class.  You must provide any necessary additions/modifications to the Easy I datapaths, flowcharts and state transition table.  The intruction has the following functionality:

| Symbolic Name | Operands | Action |
| --- | --- | --- |
| BrL | RtL sort | PC <- MEM[X-2] |

- Combinational Logic. Provide two alternative designs for a combinational circuit to control a Dice LED display.   The display has one input for each of 7 LED lights.  The combinational circuit must map three inputs encoding a binary representation of a number between 0 and 6 into the 7 control signals controlling the LED’s.  Design one should use traditional K-Map techniques and should be based on logic gates.  The second design should use a ROM or PLA.

Combinational

Logic

7

- Sequential Circuits.  Develop a 3-bit binary cyclic counter using D-Flip-Flops and connect it to the dice display developed in exercise 3 in order to make the display count as follows: 0,1, 2, 3, 4, 5, 6, 0, 1, 2, ....