¡Anota tu nombre y número de sección en todas las hojas del examen AHORA! (penalidad de 5 puntos)

Tienes 2 horas para completar todos los problemas.  Lee cuidadosamente todo el examen antes de empezar a trabajar.  Muestra todo el trabajo conducente a tu contestación.  Podrás recibir crédito parcial por contestaciones parciales siempre y cuando muestres tu trabajo por escrito.  Usa tu tiempo inteligentemente.  Exito!

INEL 4206 Staff

30

100

1

2

Total

3

25

4

10

15

20

5

Problem 1. (30 points) Finite State Machines

Consider the following finite state machine that cyclically counts by twos whenever the input signal is 1.  After counting up to six, the machine begins counting from 0 again.  For instance, suppose that the machine receives the sequence 101101.  The machine should end up in state 0, since the remainder of it increments by 2 four times.  For this input, the sequence of states traversed by the machines is:

0  2  2  4  6  6  0

The machine’s state number represents the current count of the number represented by the binary sequence of bits received so far.

- ( 5 points ) Write the sequence of states traversed by the machine for the input sequence 10101011.

- ( 5 points ) Determine the number of flip-flops necessary to implement a sequential circuit implementing the above FSM.

- (10 points)  Complete the following state transition table for the sequential circuit. The circuit should output the binary 3-bit representation of the current state number. For instance, if the machine is in state 6, the output should be 110.

| Current State Number | Current State Encoded S1S0 | Input | Next State Encoded | Outputs (3 bits) O2O1O0 |
| --- | --- | --- | --- | --- |
| 0 | 00 | 1 |  |  |
| 0 | 00 | 0 |  |  |
| 2 | 01 | 1 |  |  |
| 2 | 01 | 0 |  |  |
| 4 | 10 | 1 |  |  |
| 4 | 10 | 0 |  |  |
| 6 | 11 | 1 |  |  |
| 6 | 11 | 0 |  |  |

- (10 points) Provide Kmap and minimal Boolean equations for the high low order bit O0 of the output. Show all your work.

I

S1S0

O0====

Oo =

Problema 2.  (15 points)  Problem Set I

- (5 points) In the circuit developed inProblem Set I, ff I wanted my LED Scanner to have 8 LED’s instead of 5, what additional data and changes would I need? on the Truth Table of part 1 (Transition table)

| Input |  |  |  |  | Output |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I0 | I1 | I2 | I3 |  | O0 | O1 | O2 | O3 |
| 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 |  | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 |  | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 |  | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 |  | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 |  | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 |  | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 |  | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 |  | 0 | 0 | 0 | 1 |

- (5 points) Briefly describe the changes on the decoder.

- (5 points) Would you need more flip flops to store information for the 3 new LEDs?

Problema 3.  (25 points)  The Case of the Mysterious Easy I Program

- (15 points) The following table shows the machine code version of a mysterious Easy I program.  Your job is to reconstruct the assembly language version of the code and fill the corresponding column.  You may refer to Appendix A for information on the Easy I ISA and Appendix C for some useful binary to decimal conversions.

| Instruction Address | Assembly Code | Machine Code (binary) |
| --- | --- | --- |
| 100 |  | 0 00110 0000000000 |
| 102 |  | 0 00100 1000000000 |
| 104 |  | 0 00111 0000000110 |
| 106 |  | 0 00100 1000000010 |
| 108 |  | 0 00111 0000000001 |
| 110 |  | 0 00100 1000000100 |
| 112 | LOOP: LOADi 516 | 0 00101 1000000100 |
| 114 |  | 0 00010 0010000100 |
| 116 |  | 0 00101 1000000010 |
| 118 |  | 1 00111 1000000000 |
| 120 |  | 0 00100 1000000000 |
| 122 |  | 0 00101 1000000100 |
| 124 |  | 0 00000 dddddddddd |
| 126 |  | 0 00111 0000000001 |
| 128 |  | 0 00100 1000000100 |
| 130 |  | 0 00011 0001110000 |
| 132 | END: |  |

- (10 points) Provide a equivalent High Level Language version (C, C++, etc.) of the same program. You must name address 512 as product, address 514 as A and address 16 as B.  Warning: The last sentence may contain a hidden HINT.

Problema 4.  (20 points)  Computing Models

In class we studied the computational limits of different computing models.  For each of the problem statements below, indicate the least powerful computing model that can solve the problem and concisely justify your answer (1-2 short senteneces).  You may want to state why the less powerful models can’t solve the problem and why the model chosen can solve it.

Consider only the following models:

- Combinational logic circuits

- Sequential logic circuits

- Turing Machines

- Some model more powerful than a Turing Machine

- (5 points) Compute the sum of two N bit numbers for fixed N

- (5 points) Compute the sum of two N numbers for any N

- (5 points)  Determining is a C++ program has a syntactic error.  For instance, it is missing a curly brace } somewhere.

- (5 points) Determining if a C++ program has an infinite loop

#### Problema 5  ( 10 points)  Evaluación del curso

Es importante que completes esta parte con la mayor seriedad e interés posibles.  Tu contribución ayudará a mejorar la calidad del curso significativamente.

Gracias.

- Menciona los tres aspectos que mas te gustan de la clase INEL 4206

2) Menciona los tres aspectos que menos te gustan de la clase INEL 4206

### Appendix A. Easy I processor essentials

### Appendix B. Problem Set I

### Appendix C. Handy Decimal/Binary Conversions

| Decimal | Binary |
| --- | --- |
| 0 | 0000000000 |
| 512 | 1000000000 |
| 6 | 0000000110 |
| 514 | 1000000010 |
| 1 | 0000000001 |
| 132 | 0010000100 |
| 516 | 1000000100 |
| 112 | 0001110000 |