¡Anota tu nombre y número de sección en todas las hojas del examen AHORA! (penalidad de 5 puntos)

Tienes 2 horas para completar todos los problemas.  Lee cuidadosamente todo el examen antes de empezar a trabajar.  Muestra todo el trabajo conducente a tu contestación.  Podrás recibir crédito parcial por contestaciones parciales siempre y cuando muestres tu trabajo por escrito.  Usa tu tiempo inteligentemente.  Exito!

INEL 4206 Staff

30

40

100

1

2

Total

3

20

4

10

Problema 1. (30 puntos) High Level Language Control Structures in Assembly Language

- (10 points) Write a segment of MIPS assembly code equivalent to the following high level language code.

int a = 3;

int b = 6;

int res = 0;

main() {

if (a > b) {

res = 1;

}

else if (a < b) {

res = -1;

}

else {

res = 0;

}

}

.data			// Comments

a:		.word	3

b:		.word	6

res:		.word	0

.globl	main

.text

main:	la		$t0, a

lw		$s1, 0($t0)

lw		$s2, 4($t0)

lw		$s3, 8($t0)

blt		$s1, $s2, then

bgt		$s1, $s2, elseif

else:	li		$s3, 0

b 		end

elseif:	li		$s3, 1

b		end

then:	li		$s3, -1

end:		la		$t0, a

sw		$s1, 0($t0)

sw		$s2, 4($t0)

sw		$s3, 8($t0)

jr		$ra

- (10 points) Repeat problem 1(a) but this time generate EASY I assembly code.

// Assume a stored at address 1000

// Assume b stored at address 1002

// Assume res stored at address 1004

ANDi		0

ADDi		3

STOREi	1000

ANDi		0

ADDi		6

STOREi	1002

ANDi		0

STOREi	1004

LOADi	1000

COMP

ADDi		1

ADD		1002

BrNi		then

COMP

ADDi		1

BrN		elseif

else:	ANDi		0

JUMP		end

elseif:	ANDi		0

ADDi		1

COMP

ADDi		1

JUMP		end

then:	ANDi		0

ADDi		1

end:		STOREi	1004

- (10 points) Write a segment of MIPS assembly code equivalent to the following high level language code.

int a = 0;

int b = 6;

int res = 0;

main() {

while(a < b) {

res = res + a;

a = a + 1;

}

}

.data			// Comments

a:		.word	0

b:		.word	6

res:		.word	0

.globl	main

.text

main:	la		$t0, a

lw		$s1, 0($t0)

lw		$s2, 4($t0)

lw		$s3, 8($t0)

loop:	bge		$s1, $s2, end

add		$s3, $s3, $s1

b		loop

end:		la		$t0, a

sw		$s1, 0($t0)

sw		$s2, 4($t0)

sw		$s3, 8($t0)

jr		$ra

Problema 2.  (40 points)  Easy I Microprocessor Implementation

In this exercise, you will extend the instruction set of the Easy I processor discussed in class (refer to Appendix A for information about the Easy I) with a new instruction described in the following table:

| Symbolic Name and Description | Assembler Example | Action |
| --- | --- | --- |
| BrNri – Branch on Negative Relative Immediate | BrNri loop | AC < 0: PC  PC + 2 + X |

- ( 10 points ) Show any changes to the Easy I datapaths necessary to implement the new instruction. Show all you changes on the diagram in the next page.  Provide a concise textual description of your proposed modifications in the box below:

One possible approach is to modify the Datapaths in two ways:

1. Add path from PC to ALUB and corresponding MUX to select input B to ALU

2. Add path from AC to ABUS and corresponding MUX to select input ABUS

See next page.

Easy I Datapaths

le

A

D

D

R

E

S

S

B

U

S

A0

A bus

A

B

AC

PC

ALU

D

A

T

A

B

U

S

DI

le

op

is

sel

le

sel

sel

0

1

0

1

sel

sel

- ( 15 points )   Show a level 2 flowchart implementing the BrNr instruction.  We provide enough room for your flowchart below, but you may not need all the states depicted below.

PC + 2 -> PC

PC -> A0

brnr1

AC < 0

AC ≥ 0

ALU -> AC (ALU add)

PC -> ALUB

DI<0:9> -> ABUS -> ALUA

brnr2

AC + 2 -> ABUS -> PC

AC -> ABUS -> A0

brnr3

- ( 15 points )   Show any modifications to the Easy I state transition table described in class.  Show your changes on the table in the following page.  Also, provide a brief textual description of your changes in the box below.

Two modifications:

1. Add two columns for two new control points.  The selector for the input to the ABUS and

the selector for the input to ALUB

2. Add one row for the new brnr states

Changes on next page

Easy I State Transition Table

| Curr State | opcode | I | AC:15 |  | Next State | ALU op | Mem OP | PC sel | PC is |  | DI le | AC le | AO sel | AO le | EDB sel | ABUS sel | ALUB sel |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| reset1 | xx xxx | x | x |  | reset2 | XXX | NOP | 01 | X |  | 0 | 0 | X | 0 | X | 0 | X |
| reset2 | xx xxx | x | x |  | fetch | XXX | NOP | 10 | 1 |  | 0 | 0 | 0 | 1 | X | 0 | X |
| fetch | 00 00x | 0 | x |  | sopr | XXX | NOP | 11 | X |  | 1 | 0 | X | 0 | X | 0 | X |
| fetch | 00 010 | 0 | x |  | brn1 | XXX | RD | 11 | X |  | 1 | 0 | X | 0 | X | 0 | X |
| fetch | 00 011 | 0 | x |  | jump | XXX | RD | 11 | X |  | 1 | 0 | X | 0 | X | 0 | X |
| fetch | 00 100 | 0 | x |  | store1 | XXX | RD | 11 | X |  | 1 | 0 | X | 0 | X | 0 | X |
| fetch | 00 101 | 0 | x |  | load1 | XXX | RD | 11 | X |  | 1 | 0 | X | 0 | X | 0 | X |
| fetch | 00 11x | 0 | x |  | aopr | XXX | RD | 11 | X |  | 1 | 0 | X | 0 | X | 0 | X |
| aopr | 00 110 | x | x |  | fetch | AND | NOP | 10 | 1 |  | 0 | 1 | 0 | 1 | X | 0 | X |
| aopr | 00 111 | x | x |  | fetch | ADD | NOP | 10 | 1 |  | 0 | 1 | 0 | 1 | X | 0 | X |
| sopr | 00 000 | x | x |  | fetch | NOTB | NOP | 10 | 1 |  | 0 | 1 | 0 | 1 | X | 0 | X |
| sopr | 00 001 | x | x |  | fetch | SHRB | NOP | 10 | 1 |  | 0 | 1 | 0 | 1 | X | 0 | X |
| store1 | xx xxx | x | x |  | store2 | XXX | NOP | 11 | X | 0 |  | 0 | 1 | 1 | X | 0 | X |
| store2 | xx xxx | x | x |  | store3 | XXX | WR | 10 | 1 | 0 |  | 0 | 0 | 1 | 1 | 0 | X |
| load1 | xx xxx | x | x |  | load2 | XXX | NOP | 11 | X | 0 |  | 0 | 1 | 1 | X | 0 | X |
| load2 | xx xxx | x | x |  | load3 | XXX | RD | 11 | X | 1 |  | 0 | X | 0 | X | 0 | X |
| load3 | xx xxx | x | x |  | fetch | XXX | NOP | 10 | 1 | 0 |  | 1 | 0 | 1 | X | 0 | X |
| brn1 | xx xxx | x | 0 |  | fetch | XXX | NOP | 10 | 1 | 0 |  | 0 | 0 | 1 | X | 0 | X |
| brn1 | xx xxx | x | 1 |  | brn2 | XXX | NOP | 10 | 1 | 0 |  | 0 | 0 | 1 | X | 0 | X |
| brn2 | xx xxx | x | x |  | fetch | XXX | NOP | 10 | 0 | 0 |  | 0 | 1 | 1 | X | 0 | X |
| jump | xx xxx | x | x |  | fetch | XXX | NOP | 10 | 0 | 0 |  | 0 | 1 | 1 | X | 0 | X |
| fetch | 01000 | x | x |  | brnr1 | XXX | RD | 11 | X | 1 |  | 0 | X | 0 | X | 0 | X |
| brnr1 | xx xxx | x | 0 |  | fetch | XXX | NOP | 10 | 1 | 0 |  | 0 | 0 | 1 | X | 0 | X |
| brnr1 | xx xxx | x | 1 |  | brnr2 | XXX | NOP | 10 | 1 | 0 |  | 0 | 0 | 1 | X | 0 | X |
| brnr2 | xx xxx | x | x |  | brnr3 | ADD | NOP | 11 | X | 0 |  | 1 | X | 0 | X | 0 | 1 |
| brnr3 | xx xxx | x | x |  | fetch | XXX | NOP | 10 | 0 | 0 |  | 0 | 1 | 1 | X | 1 | X |

#### Problem 3  ( 20 points)  MIPS Assembly Code

- (10 points) Add comments to the following MIPS code and describe in one sentence what it computes. Assume that $a0 is used for the input and initially contains n, a positive integer.  Assume that $v0 is used for the output.

begin:	addi	$t0, $zero, 0		# t0 = 0

addi	$t1, $zero, 0		# t1 = 0

loop:	slt	$t2, $a0, $t1		# while (t1 <= N){

bne	$t2, $zero, finish	#

add	$t0, $t0, $t1		#     t0 += t1

addi	$t1, $t1, 3		#     t1 += 3

j	loop				# }

finish:	add	$v0, $t0, $zero	# output = $t0

#### Computes the sum all all the multiples of 3 which are less or equal to N

#### (10 points) Provide the best equivalent sequence of MIPS instructions that could be used to implement the pseudo-instruction “set on greater than or equal”:

#### sge 	$rx, $ry, $rz

#### You may use register $at for temporary results.

#### bne	$rz, $ry, nequal

#### ori	$rx, $r0, 1

#### beq	$r0, $r0, end

#### nequal:	slt	$rx, $rz, $ry

end:

#### Problema 4  ( 10 points)  Evaluación del curso

- Menciona los tres aspectos que mas te gustan de la clase INEL 4206 en orden decreciente de importancia.

- Menciona los tres aspectos que menos te gustan de la clase INEL 4206 en orden decreciente de importancia.

- En una escala de 0 (no mejoría) al 5 (mejoría excelente) como evaluarías la respuesta del professor a los comentarios que sugeriste durante el examen parcial 1. Explica brevemente.

1		2		3		4		5

Appendices

Assumes opcode for new instruction is 01000