University of Puerto Rico – Mayagüez

School of Engineering

# INEL 4206 – Microprocessors

Problem Set 1

Due: Thursday  September 5, 2002

1.	Design a combinational logic circuit that complies with the following specifications:

| Input |  |  |  |  | Output |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I0 | I1 | I2 | I3 |  | O0 | O1 | O2 | O3 |
| 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 |  | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 |  | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 1 |  | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 |  | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 |  | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 |  | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 |  | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 |  | 0 | 0 | 0 | 1 |

(This is a 01  then 1  5,  51 loop )

a)	Provide a Karnaugh Map (K-Map) and Boolean equation for each of the

outputs (On).

b) 	Make the design in Logic Works, use a “Hex Keyboard wo/STB” as input, with the pins arranged as follows:		     use Binary Probes to display the output of the circuit.

2.	Design a combinational circuit that works as a decoder, it will receive the binary equivalents for 1 through 5, and set high the output corresponding to that number.

(The circuit must have exactly 5 outputs; you may NOT use the available decoders in Logic Works)

a)	Provide the schematics for the decoder.

b) 	Make a new device symbol and package the circuit you designed above, so that it can be used as an “IC”. It should look something like this:

- Design a five LED scanner (sequence of LED’s that light up from side to side in a loop).  Use the two circuits you have designed in problems 1 and 2 as part of the sequential circuit that will make up the scanner.

Problem Set Submission:

Your solution will consist of four (4) Logic Works files.

- Solution for problem 1

- Solution for problem 2

- Library file with the packaged IC from problem 2

- Solution for problem 3

Send these 4 files in an e-mail attachment to: inel4205-submit@ece.uprm.edu

The subject of the e-mail should be    PS1:<<student #>>

K-maps and Boolean Equations from problem 1a will be handed in to the 	professor on Sept 5.

Correction criterias:

| Criteria | Weight(%) |
| --- | --- |
| Correctness | 60% |
| Design | 20% |
| Efficiency | 10% |
| Style | 10% |

Remember the policy on late submission stated on the “Course Information Sheet”.